A User Guide to BCPL

for the Raspberry Pi

D J Allerton

Department of Automatic Control and

Systems Engineering

University of Sheffield

Version 2.1

09 July 2019

Table of Contents

| 1 |  | Introduction | 3 |
| --- | --- | --- | --- |
| 2 |  | BCPL for Linux | 3 |
| 3 |  | Installation | 5 |
| 4 |  | Initialisation | 6 |
| 5 |  | Implementation | 8 |
|  | 5.1 | Registers | 8 |
|  | 5.2 | Stack frames | 9 |
| 6 |  | Procedures | 10 |
|  | 6.1 | Calling a procedure | 10 |
|  | 6.2 | Procedure entry | 11 |
|  | 6.3 | Procedure return | 12 |
|  | 6.4 | Calling C functions | 12 |
| 7 |  | Switchon statements | 13 |
| 8 |  | The code generator | 13 |
| 9 |  | An example | 14 |
| 10 |  | alib library functions | 16 |
| 11 |  | libc functions | 16 |

1	Introduction

BCPL has been implemented for a wide range of computers since the 1960s including mainframe computers, mini computers and microprocessors. A major attraction of BCPL is that the compiler is written in BCPL, helping its portability and simplifying its implementation on new computers. More recently, the use of a compact intermediate code termed Cintcode, where BCPL programs are compiled to Cintcode and interpreted, has increased the portability of BCPL.

The arrival of the Raspberry Pi, with its Arm core, network connection, video output, USB ports and I/O interface running under the Linux operating system, has encouraged use of a range of programming languages for this platform and BCPL, developed in the Computer Laboratory at the University of Cambridge, is available for the Raspberry Pi. Martin Richards’ Cintcode implementation is extensive and reliable, offering numerous examples of coding, mathematical algorithms and even operating system functions. The advantages of this implementation are considerable: it is fast to compile, reliable and robust and offers a wide range of software libraries and system functions. Consequently, for the programmer looking for an efficient and reliable implementation of BCPL, the Cintcode version is ideal. The only drawback is the loss of speed from interpreting the compiled code. An alternative approach, and the one described in this guide, is to compile directly to Arm machine code instructions, which can be linked to, and executed by, standard Linux tools.

For the programmer intending to write applications in BCPL which exploit the processing power of the Raspberry Pi, the implementation described in the guide may offer better performance. For other users, less concerned with processing speed and performance, the tools and support provided by the Cintcode implementation of BCPL offer a stable and reliable platform. The choice of implementation is left to the reader.

2	BCPL for Linux

The Raspbian version of Linux is available for the Raspberry Pi and these notes apply specifically to the Raspbian implementation. Linux provides the GNU gcc tools to support the compilation of C and C++ programs and a wide range of run-time libraries, including the libc library. Typically, a program is compiled to an object file and this file is linked with appropriate libraries to build an executable file which can be loaded and run. The implementation of BCPL for the Arm described in this guide uses a similar approach. A BCPL program prog.b is compiled to an object file prog.o which is linked with two libraries blib.o and alib.o to form an executable file prog. The main difference is that object files generated by the BCPL compiler produce position independent code, avoiding relocation.

The native code compiler is based on the compiler used for the Cintcode implementation, with minor adaptation for the Linux user interface. The interface between the compiler and code generator is fully compatible with the Cintcode version of the compiler and uses identical versions of the header files libhdr.h and bcplfecg.h.

The usual sequence to compile and run a BCPL program, for example, prog.b is:

bcpl prog.b –o prog

./prog

The first line compiles the BCPL program prog.b, generating an ELF-compatible Linux object file prog.o, which is then linked with the libraries blib.o and alib.o to form an executable file prog.

The code generator uses a minimal set of ELF functions and interested readers can inspect the contents of the generated object file, using:

readelf –a prog.o

or, examine (disassemble) the compiled code, using:

objdump –d prog.o

The command line syntax is very similar to the gcc compiler used to compile and link C programs. To compile an object file but supress linking, for example to build a library module prog.o, use:

bcpl –c prog.b

To generate an ocode file, use:

bcpl prog.b –ocode

To divert the compiler output to a file, rather than the terminal, use:

bcpl prog.b –v myfile –o prog

where the compiler output is written to the file ‘myfile’.

3	Installation

The distribution file bcpl_nm.zip (where nm denotes the version number n.m) contains the following files:

| alib.s | a run-time library written in GNU Arm assembler |
| --- | --- |
| blib.b | the BCPL run-time library, written in BCPL |
| bcpl.b | the BCPL compiler and code generator to run under Linux |
| leader.s | a small program to define the start of the BCPL sections |
| libhdr.h | the standard BCPL header |
| bcplfecg.h | a header file used by the code generator |
| bcpl.o | a precompiled version of the BCPL compiler and code generator |
| blib.o | a precompiled version of the BCPL run-time library |
| fact.b | a small program to print the factorial numbers from 1 to 5 |
| cgtest.b | a program to test the code generator |
| queens.b | an implementation of the ‘Queens’ problem for 1 to 16 pieces |
| primes.b | a small program to print the first 100 prime numbers |
| bench.b | a small test program to time execution of a small segment of BCPL |
| guide.pdf | this guide |

The object files bcpl.o and blib.o each contain a block of position independent code. The assembler program leader.s provides a means of identifying the start of a BCPL program. The assembler library alib.o contains a run-time library, including data regions for the global variables and static variables, and is linked to the GNU C run-time library libc.

Note that the files bcpl.b and bcplfecg.h are only needed to rebuild the compiler and are not required for user applications.

To install BCPL on a Raspberry Pi, create a new directory and copy the zip file to this directory. Follow the commands shown below to build and test the compiler, where the symbol > is the Linux prompt.

>unzip bcpl_nm.zip

>sudo mkdir /usr/include/BCPL

>sudo cp libhdr.h /usr/include/BCPL/

>sudo cp bcplfecg.h /usr/include/BCPL

>as leader.s –o leader.o

>as alib.s –o alib.o

>gcc leader.o bcpl.o blib.o alib.o –o bcpl

>./bcpl fact.b –o fact

>./fact

where the assembler as and the compiler gcc are provided in the GNU tool chain.

A further confidence test is to rebuild the BCPL compiler using the BCPL compiler:

>./bcpl bcpl.b –o bcpl

A more extensive test to validate the compiler is to run the compiler test program cgtest.b, which is a confidence test that the compiler and code generator are working correctly:

>./bcpl cgtest.b –o cgtest

>./cgtest

At this point, the BCPL library files and the compiler can be copied to the appropriate Linux shared directories:

>sudo cp bcpl /usr/bin

>sudo cp leader.o /usr/lib

>sudo cp blib.o /usr/lib/

>sudo cp alib.o /usr/lib

The remaining BCPL programs can now be compiled and run using the command bcpl rather than ./bcpl. The compiler looks for library files in the working directory, before searching /usr/lib.

4	Initialisation

Although a BCPL program is loaded by the Linux loader, the BCPL stack and global vector must be allocated and initialised. The entry main is located in alib and initialises the BCPL run-time environment before calling the BCPL procedure start.

At run-time, alib detects the run-time address of the first segment, enabling the initialisation code to chain through the code sections in order to initialise the global vector. The file alib.o includes a terminator that enables the initialisation code to detect the end of the executable modules produced by the gcc linker, as shown in Figure 1 (for a BCPL program comprising three sections). The linking of modules is performed by the compiler, which invokes the gcc linker to build the executable module from the sections of BCPL and the libraries.

Figure 1 Program Organisation

The module leader.o contains a label _BCPL providing the address of the first section. The first word of each section is the section size in words, which enables the run-time system to initialise the global vector, chaining from section to section until the zero pointer at the end of alib is detected. A BCPL program can contain an arbitrary number of sections and, for example, these sections can also include user libraries written in BCPL.

The structure of a section of BCPL containing three global variables and two static variables is shown in Figure 2.

section size

code

static1

static2

2

0

gn1

offset1

gn2

offset2

gn3

offset3

maxg

Figure 2 BCPL Section Structure

Initialisation of the global vector is implemented in blib which locates the start address of each section. The first word of the section is the section size, which enables the start address of the next section to be calculated. The initialisation software then searches backwards to set the nth global variable to offsetn, where the offset is the byte offset of global n from the start of the section. The zero at the head of the list is used to terminate initialisation of the global variables, where maxg is the highest global variable defined in the section. Global variables are copied to the data region gvec in alib.s where the global vector is formed as an array of n words; the contents of each entry is either the machine address of the BCPL procedure or the value unglobal, which points to code that is invoked if an undefined global procedure is called. Note that global variables are also initialised to the value unglobal.

The initialisation then searches backwards to locate the number of static variables declared in the section, where each entry (if any) contains the initial value of the static variable. For sections that contain static variables, these variables are copied from the section to the region svec in alib.s. However, it is necessary to distinguish between static variables defined in different sections. The structure of the region allocated for static variables in svec is shown in Figure 3, for a program containing two sections, where the static variables x=1, y=2 and z=3 are declared in the first section and the static variables p=100, q=200 and r=300 are declared in the second section.

| svec | 0x2000 | 0x5300 | Address of section 1 |
| --- | --- | --- | --- |
|  | 0x2004 | 0x2010 | Address of statics for section 1 |
|  | 0x2008 | 0x6324 | Address of section 2 |
|  | 0x200c | 0x201c | Address of statics for section 2 |
|  | 0x2010 | 1 | x |
|  | 0x2014 | 2 | y |
|  | 0x2018 | 3 | z |
|  | 0x201c | 100 | p |
|  | 0x2020 | 200 | q |
|  | 0x2024 | 300 | r |

Figure 3 Organisation of Static Variables

5	Implementation

This section describes the implementation of BCPL for the Raspberry Pi and is intended for users interfacing to assembler code or C libraries or developing their own libraries.

5.1	 Registers

The Raspberry Pi has sixteen 32-bit registers available in BCPL:

0	r0	data register 0

1	r1 	data register 1

2	r2 	data register 2

3	r3 	data register 3

4	r4 	data register 4

5	r5 	data register 5

6	r6 	data register 6

7	r7 	data register 7

8	r8 	data register 8

9	r9 	data register 9

10	rg	global vector

11	rp	BCPL stack

12	ip	unused

13	lr	link register

14	sp	system stack pointer

15	pc	program counter

The compiler uses registers r0 to r9 for arithmetic operations, logic operations and procedure calls. Register rg points to the global vector. Register rp is the BCPL stack pointer or frame pointer. Procedure linkage and arguments are passed during procedure calls and local variables are allocated space in the current frame. Stack space is claimed on procedure calls and released on return from a procedure. The link register lr holds the return address on entry to a procedure call and can also be used as a temporary register. The system stack pointer sp is not strictly used by the BCPL compiler, but can be used to push and pop temporary variables. The compiler uses the BCPL stack for procedure linkage and the storage of local variables. It should be noted that the Arm core is a pipelined processor and reference to the pc during an instruction implies the address of the current instruction plus eight, for most instructions. The program counter pc is used in the code generation of procedure calls, branches and switchon expressions.

5.2	Stack Frames

The stack frame, pointed to by register rp, grows upwards towards high memory. In a procedure, the first three items on the stack are:

- The previous stack pointer (the stack pointer of the calling procedure)

- The link register (the return address to the calling procedure)

- The program counter at the time of entry to the procedure (used mostly for debugging)

The arguments (if any) of the current procedure are stored immediately after the three linkage items in the current frame. Thereafter, as variables are declared in a procedure, space is allocated on the BCPL stack. On return from a procedure, the frame pointer is reinstated from the previous frame pointer at the start of the frame and the return is implemented by setting the program counter pc from the saved link register value. The stack therefore grows dynamically - upwards as procedures are called, arguments are passed and local variables are created and downwards as each procedure returns.

Figure 4 illustrates the BCPL stack for a procedure f, given by:

LET f(a, b, c) BE

{

LET p, q, r = ?, ?, ?

.

.

.

Available stack frame

FP

| 0 | fp’ |
| --- | --- |
| 4 | ra |
| 8 | pc’ |
| 12 | a |
| 16 | b |
| 20 | c |
| 24 | p |
| 28 | q |
| 32 | r |

Figure 4 Stack Organisation

The byte addresses are relative to the current frame pointer fp, starting from 0. Using Arm assembler notation, the previous frame pointer fp’ is stored at [rp, #0]. The return address ra, given by the link register lr, is stored at [rp, #4] and the current program counter pc’ (at the time of the call) is stored at [rp, #8].

The three arguments a, b and c in this example are passed in the registers r0, r1 and r2, respectively, but are also copied to locations [rp, #12], [rp, #16] and [rp, #20], respectively, on the stack. Space for the local variables p, q and r is reserved at [rp, #24], [rp, #28] and [rp, #32], respectively.

If the procedure f calls another procedure, the stack from rp+36 onwards would be used for the linkage, arguments and local variables of the called procedure.

6	Procedures

The procedure calling mechanism is specific to this implementation of BCPL and is not compatible with procedure calling mechanism used for other programming languages for the Arm processor.

6.1	Calling a Procedure

In calling a procedure, the code generator places the first four (or less) arguments in the registers r0, r1, r2 and r3, respectively. Additional arguments are copied to their appropriate locations further down the stack, so that they will be in the correct locations on entry to the called procedure. The code generator computes the new value of the stack frame pointer prior to the call and this is passed in register r4. The registers r0-r9 can be used without restriction in the called procedure. If the procedure to be called is local, a PC-relative branch and link instruction (bl) is generated, otherwise the address of the procedure is formed in register lr and a blx lr instruction is generated. For example, to call global procedure 200 with arguments in local variables [rp, #12], [rp, #16] and [rp, #20], say, the following code would be generated:

ldr r2,[rp,#20]

ldr r1,[rp,#16]

ldr r0,[rp,#12]

add r4,rp,#32

ldr lr,[rg,#800]

blx lr

The new frame pointer, given by register r4, points to the region of stack immediately above the stack storage used for local variables in the calling procedure. In this example, it is assumed there are five local variables, giving an increment of 32, or 8 words.

6.2	Procedure Entry

All BCPL procedures generated by the compiler have the following entry code, which is preceded by a BCPL string of 12 bytes containing the procedure name:

stmia   r4!,{rp,lr,pc}

stm     r4,{r0,r1,r2,r3}

sub     rp,r4,#12

In the first instruction, the registers rp, lr and pc are copied to the start of the new frame and the register r4 is incremented by 12. In the second instruction, the registers r0, r1, r2 and r3 are copied to the stack frame immediately following the saved registers rp, lr and pc, but without altering register r4. In the third instruction, the stack pointer rp is set to register r4 less 12, so that it points to the start of the current frame.

Note that all four registers r0, r1, r2 and r3 are copied independent of the number of arguments (the overhead in processor cycles is minimal). The registers r0, r1, r2 and r3 are slaved to the first four local variables and the registers r0 to r9 can be used freely within the procedure.

6.3	Procedure Return

If a procedure returns a result, it is passed in register r0. A second returned value should (by convention) be passed in the global variable result2.

All BCPL procedures execute the following return code:

ldm     rp,{rp,pc}

The previous stack pointer rp is set from the stack pointer at the base of the current frame and the program counter pc is set from the value of the link register lr, saved on entry to the procedure. Resetting the frame pointer rp, has the effect of discarding the current stack storage, freeing it for subsequent reuse in procedure calls.

6.4	Calling C functions

Although there is no explicit linkage to Linux libraries, the libc library is available to BCPL programs. Fortunately, the register calling mechanisms of the GNU gcc tool chain and BCPL are different and independent. In BCPL, the stack grows upwards and there is no access or modification to the system stack. In C, the stack grows downwards and local variables are stored relative to the system stack pointer sp.

Consequently, it is straightforward to call C functions. In the Arm APCS procedure call standard, the first four arguments are loaded into the registers r0, r1, r2 and r3, respectively, and a result is returned in register r0. The address of the procedure is computed and the procedure is called by an appropriate branch and link (bl) instruction or a branch, link and exchange instruction (blx).

However, care is needed with two specific differences between C and BCPL:

- BCPL strings are given by the string size in the first byte followed by the byte characters of the string, whereas C strings are byte characters terminated with a zero byte. BCPL strings must be converted to C strings, if calling C.

- Addresses of variables, vectors and strings in BCPL are word addresses, whereas they are machine addresses in C. Passing an address from BCPL to C requires a logical left shift of two places and passing an address from C to BCPL requires a logical right shift of two places. However, care is needed because strings in C are not necessary aligned on 32-bit word boundaries.

As with BCPL procedures, the registers r0-r9 are not preserved across procedure calls. In addition, the BCPL registers rp, rg and lr cannot be guaranteed to be preserved in C and it is advisable to store these registers before calling a C procedure. In practice, they can be pushed onto the system stack push {rg, rp, lr} and popped on return pop {rg,rp,lr}.

7	 Switchon Statements

Three methods are commonly used to code generate switchon statements in compilation:

- If there are only a few cases, simple comparisons are generated for each case.

- If a large number of cases are grouped with numerically close values, a lookup table, indexed by the case values can be implemented to provide the equivalent of a ‘computed goto’.

- The cases are ordered to enable a binary search to be implemented, affording a search time proportional to the log of the number of cases.

For more than four cases, a different approach is used for the code generator. The cases are organised as two hash tables, one for the case value and one for the corresponding destination address. The size of the hash table (in words) is the first power of two greater than (3*n)/2, where n is the number of cases, ensuring that the table is never more than two thirds full of entries. A simple linear open hash algorithm is implemented, where the hashing function uses a specific prime number in the range 13 to 251, chosen to minimise the number of possible collisions. The code to set up the hash table search is compiled in-line with a call to alib to search through the hash table and jump to the appropriate code or the default code of the switchon expression.

8	The Code Generator

The code generator makes a number of assumptions. All data registers r0-r9 can be used within procedures and the code generator attempts to keep as many variables slaved (copied) to the data registers. However, this slaving is broken for procedure calls, switchon expressions and indirect assignments, where there is no guarantee that the linkage between registers and variables could be preserved.

The Arm core has a 32-bit multiply instruction which is used in expressions with multiplication. For the Raspberry Pi Model 3, the hardware divide instruction DIVS is used, but for models without hardware divide, the operations of 32-bit division and remainder (REM) are implemented in software by a call to global procedure -1 in alib. The numerator and denominator are passed in registers r1 and r0, respectively and the result and remainder are returned in registers r0 and r1 respectively. The sign of the remainder is the same as the sign of the result. The other registers (r2-r9) are preserved across this call.

9	 An Example

The following BCPL program prints the factorial numbers 1! to 5!

SECTION "fact"

GET "libhdr"

LET start() = VALOF

{

FOR i = 1 TO 5 DO

writef("fact(%n) = %i4*n", i, fact(i))

RESULTIS 0

}

AND fact(n) = n=0 -> 1, n*fact(n-1)

The code produced by the code generator is shown with comments to explain specific instructions. Note that register r0 is reloaded at location 0x38; this is because it is reached by code from locations 0x34 and 0x74 and consequently, the content of register r0 is not assured. In addition, the reference to the string "fact(%n) = %i4*n" is not known at location 0x4C when the instruction is generated and therefore a full static reference is generated, where the offset 0x00000028 is stored at location 0x90.

0:	0000003c 	data                           section size (words)

4:	0000fddf 	data                           section identifier

8:	6361660b 	data                           section name “fact”

c:	20202074 	data

10:	20202020 	data

14:	0000dfdf 	data                           entry identifier

18:	6174730b 	data                           procedure name “start”

1c:	20207472 	data

20:	20202020 	data

24:	e8a4c800 	stmia	r4!,{fp,lr,pc}          standard procedure entry

28:	e884000f 	stm	r4,{r0,r1,r2,r3}

2c:	e244b00c 	sub	fp,r4,#12

30:	e3a00001 	mov	r0,#1                   initial value i

34:	e58b000c 	str	r0,[fp,#12]             save i

38:	e59b000c 	ldr	r0,[fp,#12]             load i

3c:	e28b4024 	add	r4,fp,#36               set new stack frame

40:	eb000017 	bl	0xa4                    call f(i)

44:	e1a02000 	mov	r2,r0                   arg 3 = f(i)

48:	e59b100c 	ldr	r1,[fp,#12]             arg 2 = i

4c:	e59fe03c 	ldr	lr,[pc,#60]             arg 1 = “fact(%n) = %i4*n”

50:	e08f000e 	add	r0,pc,lr                pc offset

54:	e1a00120 	lsr	r0,r0,#2                BCPL address

58:	e28b4010 	add	r4,fp,#16               set new stack frame

5c:	e59ae178 	ldr	lr,[sl,#376]            global writef

60:	e12fff3e 	blx	lr                      call writef()

64:	e59b000c 	ldr	r0,[fp,#12]             load i

68:	e2800001 	add	r0,r0,#1                increment by 1

6c:	e58b000c 	str	r0,[fp,#12]             store i

70:	e3500005 	cmp	r0,#5                   check end of for-loop

74:	daffffef 	ble	0x38                    back to 0x38

78:	e3a00000 	mov	r0,#0                   result = 0

7c:	e89b8800 	ldm	fp,{fp,pc}              standard return

80:	6361660f 	data                           string “fact(%n) = %i4*n”

84:	6e252874 	data

88:	203d2029 	data

8c:	0a346925 	data

90:	00000028 	data

94:	0000dfdf 	data                           entry identifier

98:	6361660b 	data                           string “fact”

9c:	20202074 	data

a0:	20202020 	data

a4:	e8a4c800 	stmia	r4!,{fp,lr,pc}          standard procedure entry

a8:	e884000f 	stm	r4,{r0,r1,r2,r3}

ac:	e244b00c 	sub	fp,r4,#12

b0:	e3500000 	cmp	r0,#0                   test n=0

b4:	1a000001 	bne	0xc0                    skip if not

b8:	e3a00001 	mov	r0,#1                   result = 1

bc:	e89b8800 	ldm	fp,{fp,pc}              standard return

c0:	e59b000c 	ldr	r0,[fp,#12]             load n

c4:	e2400001 	sub	r0,r0,#1                decrement n

c8:	e28b4010 	add	r4,fp,#16               set new stack frame

cc:	ebfffff4 	bl	0xa4                    call f(n-1)

d0:	e59b100c 	ldr	r1,[fp,#12]             get n

d4:	e0000190 	mul	r0,r0,r1                result = n*(n-1)

d8:	e89b8800 	ldm	fp,{fp,pc}              standard return

dc:	00000000	data	                        no statics

e0:	00000000	data	                        start of global vector

e4:	00000001 	data	                        global 1 (start)

e8:	00000024 	data	                        offset to global 1

ec:	0000005e 	data	                        maximum global

10	alib library functions

A number of system functions are more readily written in assembler and are provided in alib:

| level() | Returns the value of the current frame pointer (machine address) |
| --- | --- |
| longjump(p, l) | Sets the frame pointer to p and makes a non-local jump to the label l |
| muldiv(a, b, c) | Computes (a*b)/c where a*b is formed as a 64-bit intermediate product |
| selectinput(s) | Sets the current input stream (the global variable cis) to s. If s=0, the current input stream is taken from stdin in libc. |
| selectoutput(s) | Sets the current output stream (the global variable cos) to s. If s=0, the current output stream is taken from stdout in libc. |
| sys(n, args…) | Provides libc calls, where n and the argument list define the specific call, as described in section 11. |
| changeco(a, cptr) | Used to switch between coroutines. |

Three global procedures are provided for system-specific functions. These are not defined in libhdr.h as they are called by the run-time system and should not be called from a user application. Global -1 is used for software integer division if hardware division is not supported on a specific Arm architecture. Global -2 is used in the implementation of switchon statements. Global -3 is used to compute the address of a static variable.

11	libc functions

The GNU library libc is linked and loaded with the compiled program. A number of libc functions can be accessed by ‘system’ calls provided in alib, set out in the following table:

| sys call | function | used by | function |
| --- | --- | --- | --- |
| 1 | malloc(n) | getvec | dynamic memory allocation |
| 2 | free(n) | freevec | free allocated memory |
| 3 | fgetc(s) | rdch | file input |
| 4 | fputs(ch, s) | wrch | file output |
| 5 | fopen(filename, mode) | findinput, findoutput | open a file stream |
| 6 | fclose(s) | endread, endwrite | close a file stream |
| 7 | system(str) | applications | execute a process |
| 8 | gettimeofday(v) | applications | get time in secs and microsecs |

However, the equivalent BCPL functions are provided in blib and these libc functions are generally used for system tools and utilities, rather than user applications.